CHANGE No. 2 TO TENDER DOSSIER (CHANGE NOTICE)

# 2.1. General information

# Information about previous notices:

Contract Notice Title: Supply of equipment for modernisation of border crossing points in the Republic of North Macedonia
Contract Notice Reference Number (procedure identifier): GrantEU/SE_01

10. Change

Main reason for change:

Modification of original information submitted by the contracting authority.

Description: Correction of deadline for requesting clarifications from the contracting authority and Items: 12.8, 30, 31 and 32.

10.1. Text to be corrected in the original notice

Section No.: Section A, INSTRUCTIONS TO TENDERERS

Instead of:

2 Timetable

|  | DATE | TIME** |
| --- | --- | --- |
| Information meeting | 20th of February 2026 | 11:00 local time |
| Deadline for requesting clarifications from the contracting authority | 10th of March 2026 | 12:00 local time |
| Last date on which clarifications are issued by the contracting authority | 8 days before deadline for submission of tenders | - |
| Deadline for submission of tenders | As indicated in the contract notice | As indicated in the contract notice |
| Tender opening session | As indicated in the contract notice | 14:00 local time |
| Notification of award to the successful tenderer | May 2026* | - |
| Signature of the contract | May 2026* | - |

Read:

2 Timetable

|  | DATE | TIME** |
| --- | --- | --- |
| Information meeting | 20th of February 2026 | 11:00 local time |
| Deadline for requesting clarifications from the contracting authority | 17th of March 2026 | 12:00 local time |
| Last date on which clarifications are issued by the contracting authority | 8 days before deadline for submission of tenders | - |
| Deadline for submission of tenders | As indicated in the contract notice | As indicated in the contract notice |
| Tender opening session | As indicated in the contract notice | 14:00 local time |
| Notification of award to the successful tenderer | May 2026* | - |
| Signature of the contract | May 2026* | - |

Section No.: Section B, ANNEX II+III: TECHNICAL SPECIFICATIONS + TECHNICAL OFFER

Instead of:

Item 12.8: At least Windows 10 operating system preinstalled

Read:

Item 12.8: At least latest Windows operating system preinstalled

Instead of:

| 30 | Above ground Weighbridge |
| --- | --- |
|  | Quantity:5 |
|  | Manufacturer’s name: |
|  | Product type, model: |
| 30.1 | Basic requirements: |
| 30.1.1 | Minimum dimensions of 18x3 meters. |
| 30.1.2 | The weighbridge shall be raised from an elevation of + 55 cm (asphalt elevation) |
| 30.1.3 | The weighbridge shall be electronic with included management and a licensed computer system for conventional cargo motor vehicles and road traffic vehicles, approved by the Bureau of Metrology of the Republic of Macedonia. |
| 30.1.4 | The weighbridge shall have the following measurement and accuracy range: |
| 30.1.4.1 | weighting capacity from 0 kg to 50 000 kg |
| 30.1.4.2 | max 0.5% deviation |
| 30.1.4.3 | smallest notch (smallest weight increments at 20 kg max), |
| 30.1.4.4 | class (III) |
| 30.1.4.5 | Approval by the Bureau of Metrology of the Republic of North Macedonia for the weighbridge offered. |
| 30.2 | Weighbridge platform: |
| 30.2.1 | The weighbridge shall be made of one or more segments, |
| 30.2.1.2 | The weighbridge shall be raised from an elevation of + 55 cm (asphalt elevation). |
| 30.2.1.3 | from anti-corrosion protected steel parts, |
| 30.2.1.4 | with a minimum width of 3 meters and a minimum length of 18 meters, |
| 30.2.1.5 | open on both sides, |
| 30.2.1.6 | with a completely covered platform part or the platform part shall be with an opening placed in the central part of the platform, |
| 30.2.1.7 | easily accessible for servicing the system of weight sensors built into it. |
| 30.2.2 | The steel tread surface shall be made of: |
| 30.2.2.1 | a ribbed, non-slip sheet metal with high resistance to bending and yielding |
| 30.2.2.2 | to withstand a vehicle with a 15 t axle load or up to 60 t maximum load |
| 30.2.2.3 | with a steel sheet thickness of minimum 10 mm. |
| 30.2.2.4 | Steel profiles of the weighbridge structure shall be dimensioned at least with profiles U-300 and I-200 or more. |
| 30.2.3.1 | operation in a temperature range of -10°C to +40°C |
| 30.2.3.2 | resistant to rain, snow and other natural influences |
| 30.2.3.4 | the indicator shall be resistant to temperatures from -20°C to +60°C. |
| 30.3 | Construction part: |
| 30.3.1 | Foundation: |
| 30.3.1.1 | Reinforced concrete with an entrance and exit ramp with minimum C30 concrete quality |
| 30.3.1.2 | with a maximum incline of up to 6%. |
| 30.3.1.3 | with strengthened reinforced concrete places where the scale rests on the ground, |
| 30.3.1.4 | with a slight slope for atmospheric precipitation |
| 30.3.2.1 | Along the entire length of the weighbridge, protective guides (boundary-elastic fence) shall be placed for proper positioning on the weighbridge. |
| 30.3.2.2 | protective guides (boundary-elastic fence) shall have a sufficient height to protect vehicles from running off their weighbridge position. |
| 30.3.2.3 | Adequate signalling shall be installed for the proper positioning of vehicles on the weighbridge |
| 30.3.2.4 | concrete footings shall be made with minimum C30 concrete quality under the measuring cells, |
| 30.4 | System of weight sensors: |
| 30.4.1.1 | The system shall consist of at least 8 weight sensors. |
| 30.4.1.2 | the sensors must be of class C3 with a load capacity of 30 t |
| 30.4.1.3 | with protection against lightning, temperature and moisture. |
| 30.4.1.4 | system shall provide electromagnetic influence resistance |
| 30.4.1.5 | system shall provide atmospheric discharge resistance |
| 30.4.1.6 | The level of protection for sensors, connections and cables: |
| 30.4.1.6.1 | no less than MKS EN 60529:2006 IP68/IP69K or equivalent, |
| 30.4.1.6.2 | includes active spark/lightning protection of I > 80,000A, |
| 30.4.1.6.3 | cables insulated from electromagnetic influences and mechanical impacts, |
| 30.4.1.6.4 | sensors tested and certified in accordance with the Bureau of Metrology requirements. |
| 30.4.1.7 | digital indicator: |
| 30.4.1.7.1 | with minimum precision of 3000 intervals, |
| 30.4.1.7.2 | includes the self-diagnosis possibility (gives data about the scale), with data calibration memorization |
| 30.4.1.7.3 | includes a menu and software for diagnostics of scale faults, |
| 30.4.1.7.4 | has a vibration filter. |
| 30.5 | Terminal |
| 30.5.1 | With a graphic LED or LCD screen, showing measurement and diagnostic information and measurement data printing, |
| 30.5.2 | compliant with the frequency at which the Customs Administration’s system operates (2400 Megahertz) or equivalent, fully interoperable with the Customs Administration’s system |
| 30.5.3 | have at least one adequate printer port |
| 30.5.4 | have at least one Ethernet port (min 10/100Mbps) to connect to the computer where the weighbridge software will be installed |
| 30.5.5 | power supply of 220-230 VAC at 50Hz, |
| 30.5.6 | operating conditions within the range of at least from -10°C to +40°C, |
| 30.5.7 | relative humidity from 10% to 95%, |
| 30.5.8 | stainless casing material, |
| 30.5.9 | at least IP68 protection or equivalent |
| 30.6 | Weighbridge Dismantiling, Relocation. Installation and Adjustment: |
| 30.6.1 | Dismantling of the existing weighbridge |
| 30.6.2 | Relocation of the weighbridge to a location within a distance of up to 15 km |
| 30.6.3 | Installation of the weighbridge |
| 30.6.4 | Adjustment of the existing foundations for installation of the new weighbridge |
| 30.7 | The following document must be issued for the scales: |
| 30.7.1 | verification document issued by the Bureau of metrology |
| 30.7.2 | calibration certificate according to MKC EN ISO 17025 |

Read:

| 30 | Above ground Weighbridge |
| --- | --- |
|  | Quantity:5 |
|  | Manufacturer’s name: |
|  | Product type, model: |
| 30.1 | Basic requirements: |
| 30.1.1 | Minimum dimensions of 18x3 meters. |
| 30.1.2 | The weighbridge shall be raised from an elevation of at least + 30 cm (asphalt elevation) |
| 30.1.3 | The weighbridge shall have the following measurement and accuracy range: |
| 30.1.3.1 | weighting capacity from 0 kg to 50 000 kg |
| 30.1.3.2 | max 0.5% deviation |
| 30.1.3.3 | smallest notch (smallest weight increments at 20 kg max), |
| 30.1.3.4 | class (III) |
| 30.1.3.5 | Approval by the Bureau of Metrology of the Republic of North Macedonia for the weighbridge offered. |
| 30.2 | Weighbridge platform: |
| 30.2.1 | The weighbridge shall be made of two or more segments |
| 30.2.1.1 | The weighbridge shall be raised from an elevation of at least +30 cm (asphalt elevation). |
| 30.2.1.2 | Made of anti-corrosion protected steel parts, |
| 30.2.1.3 | with a minimum width of 3 meters and a minimum length of 18 meters, |
| 30.2.1.4 | open on both sides, |
| 30.2.1.5 | with a completely covered platform part or the platform part shall be with an opening placed in the central part of the platform, |
| 30.2.1.6 | easily accessible for servicing the system of weight sensors built into it. |
| 30.2.2 | The steel tread surface shall be made of: |
| 30.2.2.1 | a ribbed, non-slip sheet metal with high resistance to bending and yielding |
| 30.2.2.2 | to withstand a vehicle with a 15t axle load or up to 50t maximum load |
| 30.2.2.3 | with a steel sheet thickness of minimum 10 mm. |
| 30.2.2.4 | Steel profiles of the weighbridge structure shall be dimensioned at least with profiles U-300 and I-200 or more. |
| 30.2.3.1 | operation in a temperature range of -10°C to +40°C |
| 30.2.3.2 | resistant to rain, snow and other natural influences |
| 30.2.3.3 | the indicator shall be resistant to temperatures from -10°C to +55°C. |
| 30.3 | Construction part: |
| 30.3.1 | Foundation: |
| 30.3.1.1 | Reinforced concrete with an entrance and exit ramp with minimum C40 concrete quality |
| 30.3.1.2 | with a maximum incline of up to 6%. |
| 30.3.1.3 | with strengthened reinforced concrete places where the scale rests on the ground, |
| 30.3.1.4 | with a slight slope for atmospheric precipitation |
| 30.3.2.1 | Along the entire length of the weighbridge, protective guides (boundary-elastic fence) shall be placed for proper positioning on the weighbridge. |
| 30.3.2.2 | protective guides (boundary-elastic fence) shall have a sufficient height to protect vehicles from running off their weighbridge position. |
| 30.3.2.3 | Adequate signalling shall be installed for the proper positioning of vehicles on the weighbridge |
| 30.3.2.4 | concrete footings shall be made with minimum C40 concrete quality under the measuring cells, |
| 30.4 | System of weight sensors: |
| 30.4.1.1 | The system shall consist of at least 8 weight sensors. |
| 30.4.1.2 | the sensors must be of at least class C3 with a load capacity of at least 30t |
| 30.4.1.3 | system shall provide electromagnetic influence resistance |
| 30.4.1.4 | system shall provide atmospheric discharge resistance |
| 30.4.1.5 | sensors must have an overload capacity of 150%FS |
| 30.4.1.6 | The level of protection for sensors, connections and cables: |
| 30.4.1.6.1 | no less than MKS EN 60529:2006 at least IP67 |
| 30.4.1.6.2 | cables insulated from electromagnetic influences and mechanical impacts, |
| 30.4.1.7 | digital indicator: |
| 30.4.1.7.1 | with minimum precision of at least 3000 intervals, |
| 30.4.1.7.2 | has a vibration filter. |
| 30.5 | Terminal |
| 30.5.1 | With screen running LED, LCD or equivalent technologies, showing measurement and diagnostic information |
| 30.5.2 | Connection port compliant with the frequency at which the Customs Administration’s system operates (2400 Megahertz) or equivalent, fully interoperable with the Customs Administration’s system |
| 30.5.3 | power supply of 220-230 VAC at 50Hz, |
| 30.5.4 | operating conditions within the range of at least from -10°C to +40°C, resistant to rain, snow and other natural influences, |
| 30.5.5 | relative humidity from 10% to 80% |
| 30.5.6 | stainless casing material, |
| 30.5.7 | at least IP67 protection |
| 30.6 | Weighbridge Dismantling, Relocation. Installation and Adjustment: |
| 30.6.1 | Dismantling of the existing weighbridge |
| 30.6.2 | Relocation of the weighbridge to a location within a distance of up to 15 km |
| 30.6.3 | Installation of the weighbridge |
| 30.6.4 | Adjustment of the existing foundations for installation of the new weighbridge |

Instead of:

| 31 | Below Ground (Pit type) Weighbridge |
| --- | --- |
|  | Quantity:7 |
|  | Manufacturer’s name: |
|  | Product type, model: |
| 31.1 | Basic requirements: |
| 31.1.1 | Minimum dimensions of 18x3 meters. |
| 31.1.2 | The weighbridge shall be raised from an elevation of + - 0.00 (asphalt elevation). |
| 31.1.3 | The weighbridge shall have the following measurement and accuracy range: |
| 31.1.4 | weighting capacity from 0 kg to 50 000 kg |
| 31.1.5 | max 0.5% deviation |
| 31.1.6 | smallest notch (smallest weight increments at 20 kg max) |
| 31.1.7 | class (III) |
| 31.1.8 | Approval by the Bureau of Metrology of the Republic of North Macedonia for the weighbridge offered |
| 31.2 | Weighbridge platform: |
| 31.2.1 | The weighbridge shall be with minimum width of 3 meters and a minimum length of 18 meters, The weighbridge shall be made of tree segments with dimensions width of 3 meters, length of 6 meters and height of 0.3 m. with a completely covered platform. The segments shall be made of steel frame made of U 300 beams and filled with reinforced steel and concrete. The Frame made of anti-corrosion protected steel parts |
| 31.2.2 | easily accessible for servicing the system of weight sensors built into it. |
|  | The weighbridge shall be raised from an elevation of + - 0.00 (asphalt elevation). |
|  | The weighbridge shall have the following measurement and accuracy range: |
|  | weighting capacity from 0 kg to 50 000 kg |
|  | max 0.5% deviation |
|  | smallest notch (smallest weight increments at 20 kg max), |
|  | class (III) |
| 31.2.3 | Steel profiles for the frame of the weighbridge structure shall be dimensioned at least with profiles U-300 or more |
| 31.2.4 | operation in a temperature range of -10°C to +40°C and resistant to rain, snow and other natural influences |
| 31.2.5 | the indicator shall be resistant to temperatures from -10°C to +40°C. |
| 31.3 | Construction part: |
| 31.3.1 | Foundation: |
| 31.3.1.1 | Reinforced concrete pit with a service entrance with minimum C30 concrete quality |
| 31.3.1.2 | with strengthened reinforced concrete places where the scale rests on the ground |
| 31.3.1.3 | with a slight slope for atmospheric precipitation and atmospheric drainage |
| 31.3.1.4 | Concrete footings shall be made with minimum C30 concrete quality under the measuring cells |
| 31.3.2 | System of weight sensors: |
| 31.3.2.1 | The system shall consist of at least 8 weight sensors. |
| 31.3.2.2 | the sensors must be of class C3 with a load capacity of 30 t |
| 31.3.2.3 | sensors must have an overload capacity of 150%FS |
| 31.4 | The level of protection for sensors: |
| 31.4.1 | no less than MKS EN 60529:2006 IP68 or equivalent |
| 31.4.2 | cables insulated from electromagnetic influences and mechanical impacts |
| 31.4.3 | indicator: |
| 31.4. | with minimum precision of 3000 intervals |
| 31.4.5 | has a vibration filter |
| 31.4.6 | With a LED or LCD screen, showing measurement and information |
| 31.4.7 | Connection port RS232 compliant with the frequency at which the Customs Administration’s system operates (2400 Megahertz); or equivalent, fully interoperable with the Customs Administration’s system |
| 31.4.8 | power supply of 220-230 VAC at 50Hz |
| 31.4.9 | operating conditions within the range of at least from -10°C to +40°C, resistant to rain, snow and other natural influences, |
| 31.5 | Weighbridge Dismantiling, Relocation. and Re-Installation and Adjustment: |
| 31.5.1 | Dismantling of the existing weighbridge |
| 31.5.2 | Relocation of the weighbridge to a location within a distance of up to 15 km |
| 31.5.3 | Installation of the weighbridge |
| 31.5.4 | Adjustment of the existing foundations for installation of the new weighbridge |
| 31.6 | The following document must be issued for the scales: |
| 31.6.1 | verification document issued by the Bureau of metrology |
| 31.6.2 | calibration certificate according to MKC EN ISO 17025 |

Read:

| 31 | Below Ground (Pit type) Weighbridge |
| --- | --- |
|  | Quantity:7 |
|  | Manufacturer’s name: |
|  | Product type, model: |
| 31.1 | Basic requirements: |
| 31.1.1 | Minimum dimensions of 18x3 meters. |
| 31.1.2 | The weighbridge shall be raised from an elevation of + - 0.00 (asphalt elevation). |
| 31.1.3 | The weighbridge shall have the following measurement and accuracy range: |
| 31.1.4 | weighting capacity from 0 kg to 50 000 kg |
| 31.1.5 | max 0.5% deviation |
| 31.1.6 | smallest notch (smallest weight increments at 20 kg max) |
| 31.1.7 | class (III) |
| 31.1.8 | Approval by the Bureau of Metrology of the Republic of North Macedonia for the weighbridge offered |
| 31.2 | Weighbridge platform: |
| 31.2.1 | The weighbridge shall be made of two or more segments. |
| 31.2.2 | The Frame made of anti-corrosion protected steel parts |
| 31.2.3 | with a minimum width of 3 meters and a minimum length of 18 meters, |
| 31.2.4 | with a completely covered platform part or the platform part shall be with an opening placed in the central part of the platform, |
| 31.2.5 | easily accessible for servicing the system of weight sensors built into it. |
| 31.2.6 | The weighbridge shall be raised from an elevation of + - 0.00 (asphalt elevation). |
| 31.2.7 | The weighbridge shall have the following measurement and accuracy range: |
| 31.2.8 | weighting capacity from 0 kg to 50 000 kg |
| 31.2.9 | max 0.5% deviation |
| 31.2.10 | smallest notch (smallest weight increments at 20 kg max), |
| 31.2.11 | class (III) |
| 31.3 | The steel tread surface shall be made of: |
| 31.3.1 | a ribbed, non-slip sheet metal with high resistance to bending and yielding |
| 31.3.2 | to withstand a vehicle with a 15t axle load or up to 50t maximum load |
| 31.3.3 | with a steel sheet thickness of minimum 10 mm. |
| 31.3.4 | Steel profiles for the frame of the weighbridge structure shall be dimensioned at least with profiles U-300 and I- 200 or more |
| 31.3.5 | operation in a temperature range of -10°C to +40°C |
| 31.3.6 | resistant to rain, snow and other natural influences |
| 31.3.7 | the indicator shall be resistant to temperatures from -10°C to +55°C. |
| 31.4 | Construction part: |
| 31.4.1 | Foundation: |
| 31.4.1.1 | Reinforced concrete pit with a service entrance with minimum C40 concrete quality |
| 31.4.1.2 | with strengthened reinforced concrete places where the scale rests on the ground |
| 31.4.1.3 | with a slight slope for atmospheric precipitation and atmospheric drainage |
| 31.4.1.4 | Adequate signalling shall be installed for the proper positioning of vehicles on the weighbridge |
| 31.4.1.5 | Concrete footings shall be made with minimum C40 concrete quality under the measuring cells |
| 31.4.2 | System of weight sensors: |
| 31.4.2.1 | The system shall consist of at least 8 weight sensors. |
| 31.4.2.2 | the sensors must be at least of class C3 with a load capacity of at least 30 t |
| 31.4.2.3 | system shall provide electromagnetic influence resistance |
| 31.4.2.4 | system shall provide atmospheric discharge resistance |
| 31.4.2.5 | sensors must have an overload capacity of 150%FS |
| 31.5 | The level of protection for sensors, connections and cables:: |
| 31.5.1 | no less than MKS EN 60529:2006 at least IP67 |
| 31.5.2 | cables insulated from electromagnetic influences and mechanical impacts |
| 31.5.3 | Digital indicator: |
| 31.5.3.1 | with minimum precision of at least 3000 intervals |
| 31.5.3.2 | has a vibration filter |
| 31.6 | Terminal |
| 31.6.1 | With screen running LED, LCD or equivalent technologies, showing measurement and diagnostic information |
| 31.6.2 | Connection port compliant with the frequency at which the Customs Administration’s system operates (2400 Megahertz); or equivalent, fully interoperable with the Customs Administration’s system |
| 31.6.3 | power supply of 220-230 VAC at 50Hz |
| 31.6.4 | operating conditions within the range of at least from -10°C to +40°C, resistant to rain, snow and other natural influences, |
| 31.6.5 | relative humidity from 10% to 80%, |
| 31.6.6 | stainless casing material, |
| 31.6.7 | at least IP67 protection |
| 31.7 | Weighbridge Dismantling, Relocation. and Re-Installation and Adjustment: |
| 31.7.1 | Dismantling of the existing weighbridge |
| 31.7.2 | Relocation of the weighbridge to a location within a distance of up to 15 km |
| 31.7.3 | Installation of the weighbridge |
| 31.7.4 | Adjustment of the existing foundations for installation of the new weighbridge |

Instead of:

| 32 | Training |
| --- | --- |
| 32.1 | The Contractor shall provide training for maintenance and administration of the offered equipment for minimum 6 employees in the Customs Administration, with the minimum duration of 30 working days. Training must be in English or Macedonian language. Upon completion of the training those minimum 6 persons must gain certification for completed training. |
| 32.2 | The contractor shall provide one engineer per delivery location for field training for the period of 30 days. |
| 32.3 | If the offered equipment is from a different manufacturer than the existing VMS, the contractor must also provide: Training certificates for 6 persons employed by the bidder for the following modules already installed in the VMS: - Video Management Module - Facial Recognition module - Business intelligence report module - ANPR module - Smart Wall Module |
| 32.4 | The Contractor shall provide training materials in English or Macedonian language for installation and configuration. |

Read:

| 32 | Training |
| --- | --- |
| 32.1 | The Contractor shall provide training for maintenance and administration of the offered equipment for minimum 6 employees in the Customs Administration, with the minimum duration of 30 working days. Training must be in English or Macedonian language. Upon completion of the training those minimum 6 persons must gain certification for completed training. |
| 32.2 | The contractor shall provide training at the central location for the installed equipment on the aspects of its usage, based on approved Training plan by the Contract Authority, for at least 15 participants. |
| 32.3 | The Contractor shall provide training materials in English or Macedonian language for installation and configuration. |
| 32.4 | If the equipment proposed by the Tenderer is manufactured by an entity different from the manufacturer of the existing VMS installed at the Contracting Authority, the Tenderer shall submit documentary evidence demonstrating that at least two (2) individuals are either directly employed by, or contractually engaged with the Tenderer and are duly trained and competent in the installation, configuration, operation, and maintenance of the equipment currently deployed by the Contracting Authority, to support the installation of the new equipment. These same individuals shall hold valid and current certifications issued by: • the manufacturer of the existing installed equipment or an entity formally authorised by the manufacturer to provide certification covering the following modules already integrated into the existing VMS: ▪ Video Management Module ▪ Facial Recognition Module ▪ Business Intelligence Reporting Module ▪ Automatic Number Plate Recognition (ANPR) Module ▪ Smart Wall Module The certifications must be valid at the time of tender submission and remain valid for the entire duration of the Contract. The Contracting Authority reserves the right to request verification of the authenticity and validity of such certifications directly from the issuing manufacturer or authorised entity. |