# lANNEX II + III:	 TECHNICAL SPECIFICATIONS + TECHNICAL OFFER

Contract title: Supply of equipment for modernisation of border crossing points in the Republic of North Macedonia	p 1 /…

Publication reference: GrantEU/SE_01

Columns 1-2 should be completed by the contracting authority

Columns 3-4 should be completed by the tenderer

Column 5 is reserved for the evaluation committee

Annex III - the contractor's technical offer

The tenderers are requested to complete the template on the next pages:

- Column 2 is completed by the contracting authority shows the required specifications (not to be modified by the tenderer),

- Column 3 is to be filled in by the tenderer and must detail what is offered (for example the words ‘compliant’ or ‘yes’ are not sufficient)

- Column 4 allows the tenderer to make comments on its proposed supply and to make eventual references to the documentation

The eventual documentation supplied should clearly indicate (highlight, mark) the models offered and the options included, if any, so that the evaluators can see the exact configuration. Offers that do not permit to identify precisely the models and the specifications may be rejected by the evaluation committee.

The offer must be clear enough to allow the evaluators to make an easy comparison between the requested specifications and the offered specifications.

Unless otherwise specified, the requirements in these Technical Specifications are presented as a minimum standard which the offered goods must meet in order to be compliant. Tenderers may not submit a variant solution for the items required in these Technical Specifications. When brand names are used in the technical specification, they are “used in descriptive purposes only” since there is no other comprehensive description possible.

The tenderer is expected to submit documentary evidence (brochures, technical data sheets etc.) of the technical compliance of his offer.

1. Minimum requirements and supporting documentation

- Tenderers are required to demonstrate that the offered specifications are responsive to the Tender Dossier requirements identifying model and manufacturer of each individual item in their Technical Offer. Tenderers are to provide necessary documentation (catalogues, guides, brochures, manuals, booklets, etc.) with detailed technical specifications of all items being offered thus enabling the Contracting Authority to verify the information provided in the offer.

- Technical specifications described in the Annex III: The Contractor's technical offer, Table Offered Equipment, column 2, are minimum requirements. Tenderers may offer better specifications, with improved, additional or new features, but must not provide offers that do not strictly comply with the minimum requirements.

2. Completeness of the supply

- Supply, delivery, unloading, siting and installation, integration with existing systems, commissioning, and putting into operation must include all needed parts and accessories required for the supplies to be presented for provisional acceptance fully installed, operational and ready for use.

- Accessories, parts and documentation used during delivery, unloading, siting and installation, integration with existing systems, commissioning, and putting into operation before provisional acceptance must therefore be anticipated and calculated into the offer.

- It shall be the sole responsibility of the Tenderer to ensure that all pre-requisites for the completeness of the supply delivery are met before its commencement.

3. Supply delivery

- Installation, testing and training must be completed within the time frame set out in the Special Conditions of Supply Contracts included in the Tender Dossier. At the end of this period, all items supplied, must be fully operational and the staff chosen by the Beneficiary fully trained for safe and efficient use of the items.

- All offered equipment must be compatible and fully integrated with the existing HikCentral Video Management System (VMS).

- Provisional acceptance will depend on the supply of all the equipment, integration, and customization, as well as requested training having been satisfactorily completed.

4. Power Supply

- The equipment must have ability to be connected to the standard of Republic of North Macedonia’s single-phase power output connections.

5. General Requirements

User manuals for the equipment must be provided in English and Macedonian.

6. Installation

Equipment should be fully operational upon delivery.

7. Testing

Testing of complete equipment in duration 5 days and in the presence of representatives of the end users.

8. System Description for Automated Truck Weighing and Access Control

# As the truck approaches the weighbridge, the detection radar detects the truck and activates the green traffic light and opens the barrier. While the truck positions itself on the weighbridge, the integrated license plate recognition and axle counting camera reads the front license plate. Once the truck is properly positioned on the scale, it counts the axles and the ANPR camera installed on the barrier reads the front plate once again, while the rear ANPR camera captures the rear license plate.

# All cameras in the system are connected to a central device responsible for managing the video streams and storing both video footage and captured images. This ensures complete documentation of all events for monitoring, auditing, and security purposes.

# The weighbridge sends the recorded weight via the network to the central software system, which logs all collected data – including timestamps, front and rear license plates, axle count, and vehicle weight.

# When the driver presses the button on the ticket terminal, the printer issues a detailed ticket containing all relevant information: time of entry, front and rear license plates, truck weight, and axle count. The front barrier of the weighbridge then opens, and the driver proceeds according to the instructions shown on the information display.

# After completing all terminal procedures, the driver presents the ticket at the validation point, completes any required payments for the services used, and the system registers the front license plate in the software to authorize the exit.

As the truck approaches the exit point, the All-in-One ANPR, Barrier, and Info Display Manager recognizes the registered front plate, automatically opens the barrier, and the truck exits the terminal.

Equipment Summary

Table 1. Equipment Summary

| No. | Item (s) | Quantity |
| --- | --- | --- |
| 1. | All-in-One ANPR, Barrier and Info Display manager | 12 |
| 2. | Ticket dispenser | 18 |
| 3. | IP ANPR Camera | 36 |
| 4. | Anti fall radar | 47 |
| 5. | Trigger radar | 18 |
| 6. | Barrier gates 4m | 17 |
| 7. | Signal Light | 18 |
| 8. | Info Display | 18 |
| 9. | Terminal Server | 2 |
| 10. | NVR | 11 |
| 11. | ANPR and Axis counting camera | 6 |
| 12. | Application workstation | 23 |
| 13. | Network Switch | 11 |
| 14 | RSM Base | 1 |
| 15 | RSM Remote | 11 |
| 16. | Software License for VMS Base module and Entrance and Exit management module | 11 |
| 17. | Software License’s for ANPR Cameras for Hik Central | 31 |
| 18. | Server type I - Central Location Heartquarter | 1 |
| 19. | Server type II - Central Location Heartquarter | 2 |
| 20. | Data storage device – SAN Storage for connecting to servers | 1 |
| 21. | UPS | 11 |
| 22. | Power cable type 1 | Approx.. 3610 m |
| 23. | Power cable type 2 | Approx.. 4940 m |
| 24. | Outdoor rated FTP cable | Approx.. 8600 m |
| 25. | Plastic hose | Approx.. 8600 m |
| 26. | Metal cabinet | 11 |
| 27. | Operating system Windows server | 2 |
| 28. | Database program | 2 |
| 29. | RS232 to IP convertor | 18 |
| 30. | Above ground Weighbridge | 8 |
| 31 | Below Ground (Pit type) Weighbridge | 4 |
| 32. | Training | / |
| 33. | Warranty | / |

Equipment Distribution List

Table 2. Equipment Distribution List

| Item No. | Location/Border Crossing |  |  |  |  |  |  |  |  |  |  |  | TOTAL |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BC Bogorodica | BC Dojran | BC Novo Selo | BC Delchevo | BC Deve Bair | BC Tabanovce | BC Blace | BC Blato | BC Medzitlija | BC Stenje | BC Kjafasan | HQ |  |
| 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 0 | 11 |
| 2 | 3 | 1 | 1 | 1 | 1 | 3 | 2 | 1 | 2 | 1 | 2 | 0 | 18 |
| 3 | 6 | 2 | 2 | 2 | 2 | 6 | 4 | 2 | 4 | 2 | 6 | 0 | 38 |
| 4 | 7 | 3 | 3 | 3 | 2 | 7 | 5 | 3 | 5 | 3 | 7 | 0 | 49 |
| 5 | 3 | 1 | 1 | 1 | 1 | 3 | 2 | 1 | 2 | 1 | 3 | 0 | 19 |
| 6 | 6 | 2 | 2 | 2 | 2 | 6 | 4 | 2 | 4 | 2 | 6 | 0 | 38 |
| 7 | 3 | 1 | 1 | 1 | 1 | 3 | 2 | 1 | 2 | 1 | 3 | 0 | 19 |
| 8 | 3 | 1 | 1 | 1 | 1 | 3 | 2 | 1 | 2 | 1 | 3 | 0 | 19 |
| 9 | 1 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 2 |
| 10 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 0 | 11 |
| 11 | 3 | 0 | 0 | 0 | 0 | 3 | 0 | 0 | 0 | 0 | 0 | 0 | 6 |
| 12 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 0 | 22 |
| 13 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 0 | 11 |
| 14 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 |
| 15 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 0 | 11 |
| 16 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 0 | 11 |
| 17 | 8 | 1 | 1 | 1 | 1 | 8 | 3 | 1 | 3 | 1 | 5 | 0 | 33 |
| 18 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 |
| 19 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 2 | 2 |
| 20 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 |
| 21 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 0 | 11 |
| 22 | 570 | 190 | 190 | 190 | 190 | 570 | 380 | 190 | 380 | 190 | 570 | 0 | 3610 |
| 23 | 780 | 260 | 260 | 260 | 260 | 780 | 520 | 260 | 520 | 260 | 780 | 0 | 4940 |
| 24 | 1350 | 450 | 450 | 450 | 450 | 1350 | 900 | 450 | 900 | 450 | 1350 | 0 | 8600 |
| 25 | 1350 | 450 | 450 | 450 | 450 | 1350 | 900 | 450 | 900 | 450 | 1350 | 0 | 8600 |
| 26 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 0 | 11 |
| 27 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 2 | 2 |
| 28 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 2 | 2 |
| 29 | 3 | 1 | 1 | 1 | 1 | 3 | 2 | 1 | 2 | 1 | 3 | 0 | 19 |
| 30 | 1 | 0 | 1 | 1 | 1 | 01 | 1 | 0 | 1 | 0 | 2 | 0 | 8 |
| 31 | 0 | 1 | 0 | 1 | 0 | 0 | 1 | 1 | 0 | 0 |  | 0 | 4 |
| 32 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 |

Equipment Delivery Locations

Table 3. Equipment Delivery Locations

| No. | Location/Broder Corssing | Address (location coordinates) |
| --- | --- | --- |
| 1 | Bogorodica | 41.1345804968313, 22.550319448470443 |
| 2 | Dojran | 41.1749911107495, 22.74296710782964 |
| 3 | Novo Selo | 41.39650668546572, 22.965251083215104 |
| 4 | Delchevo | 41.987476511500525, 22.874480868320262 |
| 5 | Deve Bair | 42.220988405632994, 22.457239115630333 |
| 6 | Tabanovce | 42.23240947826488, 21.70733134883189 |
| 7 | Blace | 42.14060215711679, 21.31232673865668 |
| 8 | Blato | 41.56113208618851, 20.490436724983716 |
| 9 | Medzitlija | 40.92042883239501, 21.4147102992405 |
| 10 | Stenje | 40.92099247748571, 20.895746121814042 |
| 11 | Kjafasan | 41.09246850804845, 20.606970466937092 |

| 1. Item number | 2. Specifications required | 3. Specifications offered | 4. Notes, remarks, ref to documentation | 5. Evaluation committee’s notes |
| --- | --- | --- | --- | --- |
| 1 | All-in-One ANPR, Barrier and Info Display manager |  |  |  |
|  | Quantity: 12 |  |  |  |
|  | Manufacturer’s name: |  |  |  |
|  | Product type, model: |  |  |  |
| 1.1 | Built in ANPR camera |  |  |  |
| 1.2 | At least 4 MP resolution 2688 × 1520 |  |  |  |
| 1.3 | Sensitivity of at least 0.022 Lux illumination in color mode |  |  |  |
| 1.4 | Built-in varifocal lens size 3.1 to 6.0 mm with centralized adjustment in XYZ planes. |  |  |  |
| 1.5 | At least 25 fps at 4 MP resolution |  |  |  |
| 1.6 | Vehicle type recognition, vehicle brand recognition |  |  |  |
| 1.7 | At least 1/30s to 1/100.000 s shutter time |  |  |  |
| 1.8 | Built-in TF card with included SD card 128GB |  |  |  |
| 1.9 | TCP/IP, HTTP, DHCP, DNS, RTP, RTSP, NTP, FTP picture upload |  |  |  |
| 1.10 | JPEG with minimum 3 compression options |  |  |  |
| 1.11 | At least 2(two) RJ45 10M/100M self-adaptive Ethernet interfaces |  |  |  |
| 1.12 | At least 3-chanels triggered/alarm inputs |  |  |  |
| 1.13 | At least 2-chanels relay outputs, supporting rising/falling of barrier gate boom pole |  |  |  |
| 1.14 | At least 1(one) built-in RS-485 serial port and/or 1 (one) built-in RS-232 serial port |  |  |  |
| 1.15 | Control Supplement Light according to brightness or time |  |  |  |
| 1.16 | At least 9(nine) LED Lights |  |  |  |
| 1.17 | 3000 K Color Temperature |  |  |  |
| 1.18 | Built in LCD display that can display relevant information sent from VMS |  |  |  |
| 1.19 | At least 21 inch height |  |  |  |
| 1.20 | At least 1080p resolution |  |  |  |
| 1.21 | At least brightness 1500 cd/m² |  |  |  |
| 1.22 | Built in speaker and microphone and emergency call button |  |  |  |
| 1.23 | Built in barrier gate with 4m boom pole |  |  |  |
| 1.24 | At least 1 second barrier rising speed |  |  |  |
| 1.25 | At least 4 second barrier falling speed |  |  |  |
| 1.26 | At least 5.000.000 cycle barrier design life |  |  |  |
| 1.27 | Power supply 220v AC |  |  |  |
| 1.28 | Maximum Consumption 350W |  |  |  |
| 1.29 | At least -30 °C to 70 °C working temperature |  |  |  |
| 1.30 | At least 90% working humidity |  |  |  |
| 1.31 | Foundation slab for a ramp All in one appliance constructed from reinforced concrete |  |  |  |
| 1.32 | Replacement parts availability |  |  |  |
| 1.33 | Complete Installation, configuration and assembly in accordance with device standards and location needs |  |  |  |
| 2 | Ticket dispenser |  |  |  |
|  | Quantity: 18 |  |  |  |
|  | Manufacturer’s name: |  |  |  |
|  | Product type, model: |  |  |  |
| 2.1 | At least 10.1 inch (diagonal) LCD screen, support text, two-dimensional code and other information display, support picture, video loop playback |  |  |  |
| 2.2 | Real-time voice intercom function, Integrated echo cancellation and noise cancellation technology, all directional microphone and at least 5W speaker |  |  |  |
| 2.3 | Built in thermal printer |  |  |  |
| 2.3.1 | At least 55mm paper width |  |  |  |
| 2.3.2 | At least 120g/1㎡ paper |  |  |  |
| 2.4 | At least 1 RJ45 10M/100M self-adaptive Ethernet interfaces |  |  |  |
| 2.5 | At least 2-chanels alarm inputs |  |  |  |
| 2.6 | At least 2-chanels alarm outputs |  |  |  |
| 2.7 | At least -30 °C to 70 °C working temperature |  |  |  |
| 2.8 | At least 90% working humidity |  |  |  |
| 2.9 | Power supply 220 VAC |  |  |  |
| 2.10 | Maximum Power consumption 100W |  |  |  |
| 2.11 | Complete Installation, configuration and assembly in accordance with device standards and location needs |  |  |  |
| 3 | IP ANPR Camera |  |  |  |
|  | Quantity: 36 |  |  |  |
|  | Manufacturer’s name: |  |  |  |
|  | Product type, model: |  |  |  |
| 3.1 | At least 8 MP resolution |  |  |  |
| 3.2 | At least two (2) lens sizes 2.8 mm to 12 mm |  |  |  |
| 3.3 | At least 0.0008 Lux illumination in color mode |  |  |  |
| 3.4 | At least 1s to 1/100.000 s shutter time |  |  |  |
| 3.5 | At least 140 dB wide dynamic range |  |  |  |
| 3.6 | Built-in IR with min 60 meters distance |  |  |  |
| 3.7 | Support of at least five (5) network streams with different resolution parameters and frames |  |  |  |
| 3.8 | Minimum 20 simultaneous live view channels |  |  |  |
| 3.9 | H.265/H.264 video signal compression support |  |  |  |
| 3.10 | ONVIF Profile S,G,T,M support for integration with systems from other manufacturers |  |  |  |
| 3.11 | HTTPS encryption and logging passwords support |  |  |  |
| 3.12 | Central management and self-diagnostics |  |  |  |
| 3.13 | Built-in slot for 1 TB recording memory card with included SD card 128GB |  |  |  |
| 3.14 | At least one (1) RJ 45 10/100/1000M network interface |  |  |  |
| 3.15 | Built-in G-sensor for vibration detection |  |  |  |
| 3.16 | Support for scene change detection |  |  |  |
| 3.17 | PoE power supply |  |  |  |
| 3.18 | Working range temperature from -40 ° C to +60 ° C |  |  |  |
| 3.19 | At least IP67 protection |  |  |  |
| 3.20 | Anti Corrosion Protection - NEMA 4X (NEMA 250-2018) |  |  |  |
| 3.21 | Detection, recognition and recording registration plate, time, date, location and direction of movement (entry / exit) at least 98% accuracy regardless of the type and country of origin of the vehicle |  |  |  |
| 3.22 | Possibility of recognition vehicle type, model, brand and color |  |  |  |
| 3.23 | Bult-in camera Support for driving direction detection , |  |  |  |
| 3.24 | Possibility of fogy condition working mode |  |  |  |
| 3.25 | Heating element for lens defrosting and defogging |  |  |  |
| 3.26 | Cameras should have advanced analytical functions (alarms, predefined scenarios, searches) |  |  |  |
| 3.27 | Possibility of creation of at least 10.000 license plates records black lists |  |  |  |
| 3.27.1 | Generation of alarms under different types of criteria |  |  |  |
| 3.27.2 | Synchronization of camera blacklist with central application blacklist from central management application |  |  |  |
| 3.27.3 | At least one (1) alarm input and output for integration with other security systems |  |  |  |
| 3.27.4 | Automatic synchronization of memory card records with the NVR |  |  |  |
| 3.28 | Encryption of recorded data on the memory card |  |  |  |
| 3.29 | EN50121-4 Certificate |  |  |  |
| 3.30 | Complete installation, configuration, and assembly in accordance with device standards and site-specific requirements. The scope of work shall also include the supply and installation of mounting poles as required |  |  |  |
| 4 | Anti fall radar |  |  |  |
|  | Quantity: 47 |  |  |  |
|  | Manufacturer’s name: |  |  |  |
|  | Product type, model: |  |  |  |
| 4.1 | At least 60Ghz Transmitting frequency |  |  |  |
| 4.2 | Transmitting power less than 10mW |  |  |  |
| 4.3 | At least Vertical 10°, Horizontal 38° beam width |  |  |  |
| 4.3.1 | Maximum 50ms response time |  |  |  |
| 4.3.2 | Adjustable trigger distance from 0.3 to 6m |  |  |  |
| 4.3.3 | Adjustable horizontal detection area from 0 to 2m |  |  |  |
| 4.4 | Vehicle detection |  |  |  |
| 4.4.1 | Adjustable vehicle following area from 0.5 to 2m |  |  |  |
| 4.4.2 | Complete Installation, configuration and assembly in accordance with device standards and location needs |  |  |  |
| 5 | Trigger radar |  |  |  |
|  | Quantity: 18 |  |  |  |
|  | Manufacturer’s name: |  |  |  |
|  | Product type, model: |  |  |  |
| 5.1 | At least 60Ghz Transmitting frequency |  |  |  |
| 5.2 | Transmitting power less than 10mW |  |  |  |
| 5.3 | At least Vertical 10°, Horizontal 38° beam width |  |  |  |
| 5.4 | Maximum 50ms response time |  |  |  |
| 5.5 | Adjustable trigger distance from 0.3 to 6m |  |  |  |
| 5.6 | Adjustable horizontal detection area from 0 to 2m |  |  |  |
| 5.7 | Vehicle detection |  |  |  |
| 5.7.1 | Adjustable vehicle following area from 0.5 to 2m |  |  |  |
| 5.7.2 | Complete Installation, configuration and assembly in accordance with device standards and location needs |  |  |  |
| 6 | Barrier gates 4m |  |  |  |
|  | Quantity: 17 |  |  |  |
|  | Manufacturer’s name: |  |  |  |
|  | Product type, model: |  |  |  |
| 6.1 | At least 4 meters pole |  |  |  |
| 6.2 | Adjustable barrier raising speed 1.5 – 2.5 seconds |  |  |  |
| 6.3 | Adjustable barrier falling speed 2.5 - 3.5 seconds |  |  |  |
| 6.4 | Switchable direction of the pole |  |  |  |
| 6.5 | At least 5.000.000 cycle barrier design life |  |  |  |
| 6.6 | DC Brushless motor |  |  |  |
| 6.7 | Power supply 220 VAC |  |  |  |
| 6.8 | Maximum Consumption 100W |  |  |  |
| 6.9 | At least -30 °C to 70 °C working temperature |  |  |  |
| 6.10 | Under pole presence detection and injury protection |  |  |  |
| 6.11 | Interfaces for Rising limit, Falling limit, Open, Close, Stop, Radar, RS-485, Traffic Light |  |  |  |
| 6.12 | Complete Installation, configuration and assembly in accordance with device standards and location needs |  |  |  |
| 7 | Signal Light |  |  |  |
|  | Quantity: 18 |  |  |  |
|  | Manufacturer’s name: |  |  |  |
|  | Product type, model: |  |  |  |
| 7.1 | Red and Green signal light |  |  |  |
| 7.2 | At least 2 modules – green and red |  |  |  |
| 7.3 | At least 4000 cd/㎡ luminance |  |  |  |
| 7.4 | At least 40 lamp beads per module |  |  |  |
| 7.5 | Power supply 220 VAC |  |  |  |
| 7.6 | Maximum Consumption 7W |  |  |  |
| 7.7 | At least 300mm x 150 mm dimensions |  |  |  |
| 7.8 | At least 50.000 hours cycle design life signal lights |  |  |  |
| 7.8.1 | Complete installation, configuration, and assembly in accordance with device standards and site-specific requirements. The scope of work shall also include the supply and installation of mounting poles as required |  |  |  |
| 8 | Info Display |  |  |  |
|  | Quantity: 18 |  |  |  |
|  | Manufacturer’s name: |  |  |  |
|  | Product type, model: |  |  |  |
| 8.1 | At least 300mm width x 300 mm height/ 11,8 inch width x 11,8 inch height |  |  |  |
| 8.2 | Pixel pitch P4. 75 |  |  |  |
| 8.3 | Resolution at least 64x64 |  |  |  |
| 8.4 | Red, Green, Yellow display colors |  |  |  |
| 8.5 | At least 120° (H) / 120° (V) Viewing angle |  |  |  |
| 8.6 | Color temperature from 3000k to 6000k |  |  |  |
| 8.6.1 | At least 95% brightness uniformity |  |  |  |
| 8.6.2 | At least 95% color uniformity |  |  |  |
| 8.6.3 | At least 2000:1 contrast ratio |  |  |  |
| 8.6.4 | At least 3500 cd/m² luminance |  |  |  |
| 8.7 | Communication mode TCP |  |  |  |
| 8.8 | Working temperature -30 ° C to +60 ° C |  |  |  |
| 8.9 | Power supply 220 VAC |  |  |  |
| 8.10 | Maximum Consumption 70W |  |  |  |
| 8.11 | At least 100.000 hours cycle design life |  |  |  |
| 8.12 | Complete installation, configuration, and assembly in accordance with device standards and site-specific requirements. |  |  |  |
| 9 | Terminal Server |  |  |  |
|  | Quantity: 2 |  |  |  |
|  | Manufacturer’s name: |  |  |  |
|  | Product type, model: |  |  |  |
| 9.1 | Connection of at least 8 IP camera |  |  |  |
| 9.2 | At least 2 10/100/1000 uplink RJ 45 network interfaces |  |  |  |
| 9.3 | At least 2 optical interfaces |  |  |  |
| 9.4 | At least 4 10/100 RJ 45 network interfaces for cameras |  |  |  |
| 9.5 | At least 1 SATA hot-plug port interfaces |  |  |  |
| 9.6 | At least 4TB Video Surveillance class HDD included |  |  |  |
| 9.7 | At least 2 serial RS232 and 1 RS-485 ports |  |  |  |
| 9.8 | At least 1 USB port |  |  |  |
| 9.9 | At least 1 (one) alarm input and 1 alarm output |  |  |  |
| 9.10 | Support H.265/H.264 decoding formats |  |  |  |
| 9.11 | Power supply 220 VAC |  |  |  |
| 9.12 | Working temperature -30 ° C to +70 ° C |  |  |  |
| 9.13 | Maximum Consumption 30W |  |  |  |
| 9.14 | Fan-free design |  |  |  |
| 9.15 | Designed for roadside cabinet |  |  |  |
| 9.16 | Complete installation, configuration, and assembly in accordance with device standards and site-specific requirements. |  |  |  |
| 10 | Network Video Recorder |  |  |  |
|  | Quantity: 11 |  |  |  |
|  | Manufacturer’s name: |  |  |  |
|  | Product type, model: |  |  |  |
| 10.1 | Connection of at least 16 IP cameras |  |  |  |
| 10.2 | Support for connecting cameras with resolution of 12 MP |  |  |  |
| 10.3 | At least two (2) HDMI, two (2) VGA and one (1) video outputs |  |  |  |
| 10.4 | At least two (2) RJ 45 network interfaces |  |  |  |
| 10.5 | At least eight (8) SATA hot-plug port interfaces |  |  |  |
| 10.5.1 | At least 10TB per port interface |  |  |  |
| 10.5.2 | At least one (1) eSATA port |  |  |  |
| 10.6 | At least one (1) serial RS232 and RS-485 ports |  |  |  |
| 10.7 | At least one (1) USB 3.0 and two (2) USB 2.0 ports |  |  |  |
| 10.8 | At least eight (8) alarm inputs and four (4) alarm outputs |  |  |  |
| 10.9 | Support H.265/H.265+/H.264/MPEG4 decoding formats |  |  |  |
| 10.10 | Support for ANPR cameras |  |  |  |
| 10.11 | Dual stream recording support |  |  |  |
| 10.12 | At least 4 x 4TB Video Surveillance class HDD included |  |  |  |
| 10.13 | Complete installation, configuration, and assembly in accordance with device standards and site-specific requirements. |  |  |  |
| 11 | ANPR and Axis counting camera |  |  |  |
|  | Quantity: 6 |  |  |  |
|  | Manufacturer’s name: |  |  |  |
|  | Product type, model: |  |  |  |
| 11.1 | Built-in triple-lens camera at least 4 MP CMOS low illumination sensor. The left and right ones should have motor-driven vari-focal lenses. The middle one should have fixed-focal lens |  |  |  |
| 11.2 | Camera should be able to recognize the number of vehicle axles and tyres based on the video detection. |  |  |  |
| 11.3 | Camera should be able to capture and output the complete side view picture of the vehicle with extra long body. Supports to capture pictures of the vehicle head and tail. |  |  |  |
| 11.4 | Camera should be able to capture vehicles and corresponding functions: motor vehicles capture, LPR, and vehicle features recognition (brand and color). |  |  |  |
| 11.5 | At least 0.01 Lux illumination in color mode |  |  |  |
| 11.6 | At least lens size 4mm to 11mm for left and right camera |  |  |  |
| 11.6.1 | At least 136 ° horizontal view for central camera |  |  |  |
| 11.6.2 | At least 3.2 mm, C/S-mount fixed-focal lens for central camera |  |  |  |
| 11.7 | At least H.264 and H.265 encoding |  |  |  |
| 11.8 | At least 32 Kbps to 16 Mbps video transfer |  |  |  |
| 11.9 | At least 6 channels simultaneously live view |  |  |  |
| 11.10 | At least 1 (one) RJ45 10 M/100 M/1000 M ethernet interface |  |  |  |
| 11.11 | At least 1 (one) RS-485 interface |  |  |  |
| 11.12 | At least 2 (two) × IO input interface |  |  |  |
| 11.13 | Micro SD Card slot with included SD card 128GB |  |  |  |
| 11.14 | Camera should be preinstalled on a pole with height of at least 1800mm and diameter of 140mm |  |  |  |
| 11.15 | Working temperature -30 ° C to +70 ° C |  |  |  |
| 11.16 | Power supply 220Vac |  |  |  |
| 11.17 | Maximum 60W consumption |  |  |  |
| 11.18 | Complete installation, configuration, and assembly in accordance with device standards and site-specific requirements. |  |  |  |
| 12 | Application workstation |  |  |  |
|  | Quantity: 23 |  |  |  |
|  | Manufacturer’s name: |  |  |  |
|  | Product type, model: |  |  |  |
| 12.1 | Processor i7-12700 (2.1 GHz-4.9 GHz, 25 MB Cache， 12 Cores, 20 Threads ) |  |  |  |
| 12.2 | At least 16 GB DDR RAM |  |  |  |
| 12.3 | HDD At least 1 TB M.2 SSD e |  |  |  |
| 12.4 | At least one (1) RJ 45 network interface |  |  |  |
| 12.5 | Power supply 220 VAC |  |  |  |
| 12.6 | At least 1(one) HDMI port |  |  |  |
| 12.7 | At least 3 (three) DP ports |  |  |  |
| 12.8 | At least Windows 10 operating system preinstalled |  |  |  |
| 12.9 | At least 1 (one) USB C and 4 (four) USB 3 ports |  |  |  |
| 12.10 | Complete installation, configuration, and assembly in accordance with device standards and site-specific requirements. |  |  |  |
| 13 | Network Switch |  |  |  |
|  | Quantity: 11 |  |  |  |
|  | Manufacturer’s name: |  |  |  |
|  | Product type, model: |  |  |  |
| 13.1 | At least twenty-four (24) Gbps PoE network interface |  |  |  |
| 13.2 | At least two (2) Gps fiber optical network interface |  |  |  |
| 13.3 | Network topology management |  |  |  |
| 13.4 | Alarm push |  |  |  |
| 13.5 | Network health monitor |  |  |  |
| 13.6 | PoE watchdog to auto detect and restart the cameras that do not respond |  |  |  |
| 13.7 | Support of Layer 2 management protocols, STP/RSTP, VLAN, link aggregation, SNMP, QoS |  |  |  |
| 13.8 | At least 300 meters PoE functionality |  |  |  |
| 13.9 | PoE watchdog functionality |  |  |  |
| 13.10 | Fully compatible with the central software application |  |  |  |
| 13.11 | 6 KV surge protection on PoE ports. |  |  |  |
| 13.12 | Power supply 220 VAC |  |  |  |
| 13.13 | Working range temperature from -10 ° C to +50 ° C |  |  |  |
| 13.14 | Complete installation, configuration, and assembly in accordance with device standards and site-specific requirements. |  |  |  |
| 14 | Software License for VMS Base module for Remote Site Managment |  |  |  |
|  | Quantity: 1 |  |  |  |
|  | Manufacturer’s name: |  |  |  |
|  | Product type, model: |  |  |  |
| 14.1 | Base license for remote server management |  |  |  |
| 14.2 | Compatible with existing HikCentral Professional or equivalent |  |  |  |
| 15 | Software License’s for Remote Site Server |  |  |  |
|  | Quantity: 11 |  |  |  |
|  | Manufacturer’s name: |  |  |  |
|  | Product type, model: |  |  |  |
| 15.1 | License for remote server connection |  |  |  |
| 16 | Software License for VMS Base module and Entrance and Exit management module |  |  |  |
|  | Quantity: 11 |  |  |  |
|  | Manufacturer’s name: |  |  |  |
|  | Product type, model: |  |  |  |
| 16.1 | Compatible with Existing VMS Hik Central professional or equivalent |  |  |  |
| 16.2 | Entrance and Exit management module |  |  |  |
| 16.3 | Management of Display Screens |  |  |  |
| 16.4 | Calling center from ticket dispenser, all in one entrance / exit units |  |  |  |
| 16.5 | Barrier control |  |  |  |
| 16.6 | Rule management |  |  |  |
| 16.7 | Entrance and Exit monitoring an operation |  |  |  |
| 16.8 | Vehicle list management |  |  |  |
| 17 | Software Licenses for ANPR Cameras |  |  |  |
|  | Quantity: 31 |  |  |  |
|  | Manufacturer’s name: |  |  |  |
|  | Product type, model: |  |  |  |
| 17.1 | Expansion license for 1 Lane and 1 ANPR camera |  |  |  |
| 18 | Server type I - Central Location Head quarter |  |  |  |
|  | Quantity: 1 |  |  |  |
|  | Manufacturer’s name: |  |  |  |
|  | Product type, model: |  |  |  |
| 18.1 | Minimum one processor minimum 8 cores and 16 threads, base frequency of minimum 2.7 GHz, 24 MB Cache with Hyper-Threading. |  |  |  |
| 18.2 | Minimum 32GB DDR5 5600 MHz dual rank memory with upgradeable to 128 GB |  |  |  |
| 18.3 | Minimum of 2 x 960 GB SSD SATA M.2 drives |  |  |  |
| 18.4 | Integrated graphics with 16mb memory with the ability to support a resolution of 1920x1200 and support 1 graphics unit with a minimum of 8GB PCIe Gen4 |  |  |  |
| 18.5 | Front: Minimum 1 x USB 2.0 Type-A and 1 x USB 3.2 gen 1 port, possibility of a VGA port. |  |  |  |
| 18.6 | Rear: Minimum 2 x USB 3.2 gen 1 and 1 x VGA port; possibility of 1 x serial port. |  |  |  |
| 18.7 | Should have the ability to access the management controller through at least 1 (one) USB port with dual functionality from the front, must have dedicated network port from the rear that allow redundancy. |  |  |  |
|  | At least 2 x 1 Gb Base-T ports supporting 10/100 Mb |  |  |  |
|  | Redundant and Hot-Plug power modules of at least 800W power. |  |  |  |
|  | Included power cables from a standard PDU – C13/C14 from the manufacturer, with a length of at least 2m |  |  |  |
|  | The server must be NIST SP800-193 compliant. |  |  |  |
|  | Rack (1(one) Unit) |  |  |  |
|  | Complete installation, configuration, and assembly in accordance with device standards in Data Centar |  |  |  |
| 19 | Server type II - Central Location Heartquarter |  |  |  |
|  | Quantity: 2 |  |  |  |
|  | Manufacturer’s name: |  |  |  |
|  | Product type, model: |  |  |  |
| 19.1 | 1 (one) Unit height server |  |  |  |
| 19.2 | Minimum one processor minimum 16 cores and 32 threads, base frequency of minimum 2.0GHz, 30 MB Cache with Hyper-Threading and Intel Deep Learning Boost technology |  |  |  |
| 19.3 | Possibility to upgrade with a second processor |  |  |  |
| 19.4 | Minimum 32GB DDR5 5600 MHz dual rank RDIMM memory with upgradeable to 8 TB memory via 32x 256 GB RDIMM memories |  |  |  |
| 19.5 | Minimum 2 x 10 GbE Base-T ports supporting 10Gb/1Gb speed. |  |  |  |
| 19.5.1 | Minimum 2 x 16Gb Fiber Channel ports |  |  |  |
| 19.5.2 | Мust support 8 x 2.5” drives with the ability to install the following types: SAS, SATA, SSD and 16 EDSFF drives |  |  |  |
| 19.5.3 | Мinimum of 2 x 240 GB SSD SATA M.2 drives |  |  |  |
| 19.5.4 | Hardware RAID controller that supports SATA and NVMe, RAID levels RAID 0, 1 with the ability for JBOD functionality |  |  |  |
| 19.5.5 | Integrated graphics with 16mb memory with 2D hardware accelerator with the ability to support a resolution of 1920x1200 and support 3 graphics units each with a minimum of 24GB GDDR, PCIe Gen4 or higher |  |  |  |
| 19.5.6 | Front: Minimum 1 x USB 2.0 and 1 x USB 3.2 gen 1 port, possibility of a VGA port. |  |  |  |
| 19.5.7 | Rear: Minimum 3 x USB 3.2 gen 1 and 1 x VGA port; possibility of 1 x DB-9 serial port. |  |  |  |
| 19.5.8 | should have the ability to access the management controller through at least 1 (one) USB port with dual functionality from the front and the ability (optional) for two dedicated network ports from the rear that allow redundancy. |  |  |  |
| 19.5.9 | must also have an internal USB 3.2 gen 1 that can be used for the Operating System or for license keys. |  |  |  |
| 19.5.10 | Minimum 5 x PCI slots of which at least 2 x PCIe 5.0, |  |  |  |
| 19.5.11 | Built-in system management controller via a dedicated network port and at least one USB 2.0 port (on the front) that can have two operating modes (for management and USB mode), through the same USB port it is necessary to be able to Power on, power off and restart the server, to be able to set network stops as well as check the validity of the warranty. Possibility of managing the server with an application from the manufacturer by physically connecting an IOS or Android device to the front USB port. The solution should also enable Quality/Bandwidth control during remote access to the server. The controller should enable system monitoring, alerting and notifications, firmware upgrades and performance monitoring as well as real-time power usage, remote installation of the operating system, remote video display and mapping of ISO files via HTTPS, Samba and NFS as well as collaboration of at least 6 users with a virtual console. The controller should support Web, IPMI, SNMPv3, CLI Support for Microsoft Windows Server 2019, 2022, 2025; Red Hat Enterprise Linux 9.5; SUSE Linux Enterprise Server 15; VMware vSphere (ESXi) 7.0, 8.0 and 8.0 U3 |  |  |  |
| 19.5.12 | Redundant and Hot-Plug power modules of at least 750W power. |  |  |  |
| 19.5.13 | Included power cables from a standard PDU – C13/C14 from the manufacturer, with a length of at least 2m. |  |  |  |
| 19.5.14 | Мust be certified for energy efficiency according to the Energy Star program. |  |  |  |
| 19.5.15 | Мust be delivered with a minimum of 6 x Hot-Swap CPU coolers and TPM 2.0 |  |  |  |
| 19.5.16 | The server must be NIST SP800-193 compliant. |  |  |  |
| 19.5.17 | The offered server must be delivered with a diagnostic port on the front of the server and for this it is necessary to provide an appropriate device that, through the control buttons, will allow access to the status of the server itself, firmware version, IP address of the management controller, temperature of the processor itself, as well as estimated energy consumption. |  |  |  |
|  | Complete installation, configuration, and assembly in accordance with device standards in Data Centar |  |  |  |
| 20 | Data storage device – SAN Storage for connecting to servers |  |  |  |
|  | Quantity: 1 |  |  |  |
|  | Manufacturer’s name: |  |  |  |
|  | Product type, model: |  |  |  |
| 20.1 | Rackmount hybrid device with a size of 2U and appropriate rails for a standard cabinet |  |  |  |
| 20.2 | Minimum configuration with two fully redundant controllers operating in "active-active" mode. In the event of a failure of one controller, the other must automatically take over the functions of the inactive one. Minimum 12 cores (6 processor cores per controller). If the offered system has fewer cores, a sufficient number should be offered to transfer this request. |  |  |  |
| 20.3 | 4 (four) pieces of SAS SSD modules of 1.92 TB 12Gb 2.5 inches. |  |  |  |
| 20.4 | Possibility of expansion with 6 (six)x 2 (two) Units cabinets through which the system will have 440 disk, or 880 disks in clustered system. |  |  |  |
| 20.5 | The connection to the disk office should be via a SAS connection |  |  |  |
| 20.6 | Minimum 64 GB RAM cache for the entire system, implemented without the use of SSD/Flash drives, added controllers or virtualization. |  |  |  |
| 20.7 | NSPOF system architecture with redundant components in the device subsystems. |  |  |  |
| 20.8 | The following RAID configurations are supported as a minimum: DRAID 1, 5 and 6 |  |  |  |
| 20.9 | Supported disk technologies: SAS, SAS SSD and NL-SAS |  |  |  |
| 20.10 | The system must support disk encryption and "data encryption at rest". Functionality activated with an additional license |  |  |  |
| 20.11 | Creation of a copy "snap, mirror, volume" must be supported without the use of user devices(The functionality must be licensed for the full capacity of the storage system). |  |  |  |
| 20.12 | Encryption functionality ("data encryption at rest") must be available. The system should control Data reduce pools with in provisioning, compression and deduplication. |  |  |  |
| 20.13 | Data replication must be supported at the device level. Replication must be supported in synchronous and asynchronous modes of operation over Ethernet or FC connections. The replication topology must have replication from one to many sites, as well as from many sites to one site in cases of unforeseen situations ("disaster recovery"). |  |  |  |
| 20.14 | The storage system must have the ability to quickly deploy storage services in a block structure for both traditional workloads, on-premises (at the user's location), off-premises (in the Cloud), and a combination of the two; |  |  |  |
| 20.15 | It must support higher-level operating systems: Microsoft Windows, VMware ES, Linux, IBM AIX, HP-UX, SUN Solaris or equivalent. |  |  |  |
| 20.16 | It must control/integrate with the "OpenStack" system. |  |  |  |
| 20.17 | It must have data storage and integration for private "Cloud" containerization ("Container Storage Interface"). |  |  |  |
| 20.18 | The device must be managed via WEB, command line, or REST API. |  |  |  |
| 20.19 | The system must support virtualization of external storage from various manufacturers without the use of additional devices. In this way, data migration between different storage systems must be supported without interruption during online operation. |  |  |  |
| 20.20 | The storage system must have the option to configure the “Call Home” function, for automatic reporting of early indications of failures for support, and as an incident diagnostic. Minimum four (two per controller) 10Gb RJ-45 Ethernet ports as standard (“built-in”) for iSCSI connectivity and minimum eight (four per controller) 16Gb FiberChannel ports with designated SPF. |  |  |  |
| 20.21 | Dual power connections to both control modules and control unit-extension enclosures (redundant power supply). |  |  |  |
| 20.22 | Complete installation, configuration, and assembly in accordance with device standards in Data Centar |  |  |  |
| 21 | UPS |  |  |  |
|  | Quantity: 11 |  |  |  |
|  | Manufacturer’s name: |  |  |  |
|  | Product type, model: |  |  |  |
| 21.1 | At least 3000VA / 2700W power capacity |  |  |  |
| 21.2 | At least 110 VAC to 300 VAC input voltage range |  |  |  |
| 21.3 | At least 0.9 output power factor |  |  |  |
| 21.4 | Less than 3 % THD in linear load |  |  |  |
| 21.5 | Rackmount cabinet installation |  |  |  |
| 21.6 | Working range temperature from 0 ° C to 40 ° C |  |  |  |
| 21.7 | Complete installation, configuration, and assembly in accordance with device standards and site-specific requirements. |  |  |  |
| 22 | Power cable type 1 |  |  |  |
|  | Quantity: approx. 3610m |  |  |  |
|  | Manufacturer’s name: |  |  |  |
|  | Product type, model: |  |  |  |
| 22.1 | Cooper PVC insulated and shielded cable |  |  |  |
| 22.2 | Type 3 x 1.5 mm2 PP/Y |  |  |  |
| 22.3 | 200/300 VAC nominal voltage |  |  |  |
| 23 | Power cable type 2 |  |  |  |
|  | Quantity: approx. 4940m |  |  |  |
|  | Manufacturer’s name: |  |  |  |
|  | Product type, model: |  |  |  |
| 23.1 | Cooper PVC insulated and shielded cable |  |  |  |
| 23.2 | Type 3 x 2.5 mm2 PP/Y |  |  |  |
| 23.3 | 200/300 VAC nominal voltage |  |  |  |
| 24 | Outdoor rated FTP cable |  |  |  |
|  | Quantity: approx. 8600m |  |  |  |
|  | Manufacturer’s name: |  |  |  |
|  | Product type, model: |  |  |  |
| 24.1 | FTP (F/UTP) cable 4x2xAWG24, Category 5E, 300 MHz |  |  |  |
| 24.2 | If it is used for PoE, copper wire usage is preferred with no alloys. |  |  |  |
| 24.3 | Shielded with AL/PET foil |  |  |  |
| 24.4 | At least 300 MHz tested bandwidth |  |  |  |
| 24.5 | At least 110N tensile load |  |  |  |
| 24.6 | Resistant to moisture, water and UV radiation |  |  |  |
| 24.7 | Resistant to mechanical damage and environmental influences |  |  |  |
| 24.8 | Possibility of mounting RJ45 connectors directly on the cable |  |  |  |
| 24.9 | At least -20 °C to +60 °C operating temperature |  |  |  |
| 25 | Plastic hose |  |  |  |
|  | Quantity: approx. 8600m |  |  |  |
|  | Manufacturer’s name: |  |  |  |
|  | Product type, model: |  |  |  |
| 25.1 | Flexible PVC Corrugated Cable Hose |  |  |  |
| 25.2 | At least 20 mm diameter |  |  |  |
| 25.3 | UV resistant |  |  |  |
| 26 | Metal cabinet |  |  |  |
|  | Quantity: 11 |  |  |  |
|  | Manufacturer’s name: |  |  |  |
|  | Product type, model: |  |  |  |
| 26.1 | 15U rack delivered with a complete set of required cables, patch cords, power cords, Ethernet cables, accessories and other physical modules required for the proposed functions according to technical specification |  |  |  |
| 26.2 | At least width 600mm, depth 600mm, height 770mm |  |  |  |
| 26.3 | Produced from quality cold rolled steel |  |  |  |
| 26.4 | Removable side panels, wire way top and bottom |  |  |  |
| 26.5 | Wall installation |  |  |  |
| 26.6 | 1(one) Unit Metal shelf 450mm depth |  |  |  |
| 26.7 | 2 (two) pcs. x 1(one) Unit Aluminum PDU, rack mountable, 6(six) x safety outlet 16A Plug, 250VAC 50/60Hz, switch, surge Protection |  |  |  |
| 26.8 | 2 (two) pcs. x 1(one) Unit cable management with cap |  |  |  |
| 26.9 | 1(one) pcs. x CAT 5 patch panel, shielded, 24-port RJ45, 1 (one) Unit, rack mount |  |  |  |
| 26.10 | 12 (twelve) pcs. x SF/UTP Cat5e PVC RJ45 0.5m patch cable |  |  |  |
| 26.11 | 2(two) pcs. x SF/UTP Cat5e PVC RJ45 2m patch cable |  |  |  |
| 26.12 | Complete installation, configuration, and assembly in accordance with device standards and site-specific requirements. |  |  |  |
| 27 | Operating system Windows server |  |  |  |
|  | Quantity: 2 |  |  |  |
|  | Manufacturer’s name: |  |  |  |
|  | Product type, model: |  |  |  |
| 27.1 | Windows server 2022 Standard or equivalent, with support for a minimum of 16 cores |  |  |  |
| 28 | Database program |  |  |  |
|  | Quantity: 2 |  |  |  |
|  | Manufacturer’s name: |  |  |  |
|  | Product type, model: |  |  |  |
| 28.1 | Microsoft SQL Server 2022 Standard, with access CALs included or equivalent |  |  |  |
| 29 | RS232 to IP Converter for weighbridge |  |  |  |
|  | Quantity: 18 |  |  |  |
|  | Manufacturer’s name: |  |  |  |
|  | Product type, model: |  |  |  |
| 29.1 | At least 1 (one) RS232 input port |  |  |  |
| 29.2 | Support for static IP address |  |  |  |
| 29.3 | Complete installation, configuration, and assembly in accordance with device standards and site-specific requirements. |  |  |  |
| 30 | Above ground Weighbridge |  |  |  |
|  | Quantity:8 |  |  |  |
|  | Manufacturer’s name: |  |  |  |
|  | Product type, model: |  |  |  |
| 30.1 | Basic requirements: |  |  |  |
| 30.1.1 | Minimum dimensions of 18x3 meters. |  |  |  |
| 30.1.2 | The weighbridge shall be raised from an elevation of + 55 cm (asphalt elevation) |  |  |  |
| 30.1.3 | The weighbridge shall be electronic with included management and a licensed computer system for conventional cargo motor vehicles and road traffic vehicles, approved by the Bureau of Metrology of the Republic of Macedonia. |  |  |  |
| 30.1.4 | The weighbridge shall have the following measurement and accuracy range: |  |  |  |
| 30.1.4.1 | weighting capacity from 0 kg to 50 000 kg |  |  |  |
| 30.1.4.2 | max 0.5% deviation |  |  |  |
| 30.1.4.3 | smallest notch (smallest weight increments at 20 kg max), |  |  |  |
| 30.1.4.4 | class (III) |  |  |  |
| 30.1.4.5 | Approval by the Bureau of Metrology of the Republic of North Macedonia for the weighbridge offered. |  |  |  |
| 30.2 | Weighbridge platform: |  |  |  |
| 30.2.1 | The weighbridge shall be made of one or more segments, |  |  |  |
| 30.2.1.2 | The weighbridge shall be raised from an elevation of + 55 cm (asphalt elevation). |  |  |  |
| 30.2.1.3 | from anti-corrosion protected steel parts, |  |  |  |
| 30.2.1.4 | with a minimum width of 3 meters and a minimum length of 18 meters, |  |  |  |
| 30.2.1.5 | open on both sides, |  |  |  |
| 30.2.1.6 | with a completely covered platform part or the platform part shall be with an opening placed in the central part of the platform, |  |  |  |
| 30.2.1.7 | easily accessible for servicing the system of weight sensors built into it. |  |  |  |
| 30.2.2 | The steel tread surface shall be made of: |  |  |  |
| 30.2.2.1 | a ribbed, non-slip sheet metal with high resistance to bending and yielding |  |  |  |
| 30.2.2.2 | to withstand a vehicle with a 15 t axle load or up to 60 t maximum load |  |  |  |
| 30.2.2.3 | with a steel sheet thickness of minimum 10 mm. |  |  |  |
| 30.2.2.4 | Steel profiles of the weighbridge structure shall be dimensioned at least with profiles U-300 and I-200 or more. |  |  |  |
| 30.2.3.1 | operation in a temperature range of -10°C to +40°C |  |  |  |
| 30.2.3.2 | resistant to rain, snow and other natural influences |  |  |  |
| 30.2.3.4 | the indicator shall be resistant to temperatures from -20°C to +60°C. |  |  |  |
| 30.3 | Construction part: |  |  |  |
| 30.3.1 | Foundation: |  |  |  |
| 30.3.1.1 | Reinforced concrete with an entrance and exit ramp with minimum C30 concrete quality |  |  |  |
| 30.3.1.2 | with a maximum incline of up to 6%. |  |  |  |
| 30.3.1.3 | with strengthened reinforced concrete places where the scale rests on the ground, |  |  |  |
| 30.3.1.4 | with a slight slope for atmospheric precipitation |  |  |  |
| 30.3.2.1 | Along the entire length of the weighbridge, protective guides (boundary-elastic fence) shall be placed for proper positioning on the weighbridge. |  |  |  |
| 30.3.2.2 | protective guides (boundary-elastic fence) shall have a sufficient height to protect vehicles from running off their weighbridge position. |  |  |  |
| 30.3.2.3 | Adequate signalling shall be installed for the proper positioning of vehicles on the weighbridge |  |  |  |
| 30.3.2.4 | concrete footings shall be made with minimum C30 concrete quality under the measuring cells, |  |  |  |
| 30.4 | System of weight sensors: |  |  |  |
| 30.4.1.1 | The system shall consist of at least 8 weight sensors. |  |  |  |
| 30.4.1.2 | the sensors must be of class C3 with a load capacity of 30 t |  |  |  |
| 30.4.1.3 | with protection against lightning, temperature and moisture. |  |  |  |
| 30.4.1.4 | system shall provide electromagnetic influence resistance |  |  |  |
| 30.4.1.5 | system shall provide atmospheric discharge resistance |  |  |  |
| 30.4.1.6 | The level of protection for sensors, connections and cables: |  |  |  |
| 30.4.1.6.1 | no less than MKS EN 60529:2006 IP68/IP69K or equivalent, |  |  |  |
| 30.4.1.6.2 | includes active spark/lightning protection of I > 80,000A, |  |  |  |
| 30.4.1.6.3 | cables insulated from electromagnetic influences and mechanical impacts, |  |  |  |
| 30.4.1.6.4 | sensors tested and certified in accordance with the Bureau of Metrology requirements. |  |  |  |
| 30.4.1.7 | digital indicator: |  |  |  |
| 30.4.1.7.1 | with minimum precision of 3000 intervals, |  |  |  |
| 30.4.1.7.2 | includes the self-diagnosis possibility (gives data about the scale), with data calibration memorization |  |  |  |
| 30.4.1.7.3 | includes a menu and software for diagnostics of scale faults, |  |  |  |
| 30.4.1.7.4 | has a vibration filter. |  |  |  |
| 30.5 | Terminal |  |  |  |
| 30.5.1 | With a graphic LED or LCD screen, showing measurement and diagnostic information and measurement data printing, |  |  |  |
| 30.5.2 | compliant with the frequency at which the Customs Administration’s system operates (2400 Megahertz) or equivalent, fully interoperable with the Customs Administration’s system |  |  |  |
| 30.5.3 | have at least one adequate printer port |  |  |  |
| 30.5.4 | have at least one Ethernet port (min 10/100Mbps) to connect to the computer where the weighbridge software will be installed |  |  |  |
| 30.5.5 | power supply of 220-230 VAC at 50Hz, |  |  |  |
| 30.5.6 | operating conditions within the range of at least from -10°C to +40°C, |  |  |  |
| 30.5.7 | relative humidity from 10% to 95%, |  |  |  |
| 30.5.8 | stainless casing material, |  |  |  |
| 30.5.9 | at least IP68 protection or equivalent |  |  |  |
| 30.6 | Weighbridge Dismantiling, Relocation. Installation and Adjustment: |  |  |  |
| 30.6.1 | Dismantling of the existing weighbridge |  |  |  |
| 30.6.2 | Relocation of the weighbridge to a location within a distance of up to 15 km |  |  |  |
| 30.6.3 | Installation of the weighbridge |  |  |  |
| 30.6.4 | Adjustment of the existing foundations for installation of the new weighbridge |  |  |  |
| 30.7 | The following document must be issued for the scales: |  |  |  |
| 30.7.1 | verification document issued by the Bureau of metrology |  |  |  |
| 30.7.2 | calibration certificate according to MKC EN ISO 17025 |  |  |  |
| 31 | Below Ground (Pit type) Weighbridge |  |  |  |
|  | Quantity:4 |  |  |  |
|  | Manufacturer’s name: |  |  |  |
|  | Product type, model: |  |  |  |
| 31.1 | Basic requirements: |  |  |  |
| 31.1.1 | Minimum dimensions of 18x3 meters. |  |  |  |
| 31.1.2 | The weighbridge shall be raised from an elevation of + - 0.00 (asphalt elevation). |  |  |  |
| 31.1.3 | The weighbridge shall have the following measurement and accuracy range: |  |  |  |
| 31.1.4 | weighting capacity from 0 kg to 50 000 kg |  |  |  |
| 31.1.5 | max 0.5% deviation |  |  |  |
| 31.1.6 | smallest notch (smallest weight increments at 20 kg max) |  |  |  |
| 31.1.7 | class (III) |  |  |  |
| 31.1.8 | Approval by the Bureau of Metrology of the Republic of North Macedonia for the weighbridge offered |  |  |  |
| 31.2 | Weighbridge platform: |  |  |  |
| 31.2.1 | The weighbridge shall be with minimum width of 3 meters and a minimum length of 18 meters, The weighbridge shall be made of tree segments with dimensions width of 3 meters, length of 6 meters and height of 0.3 m. with a completely covered platform. The segments shall be made of steel frame made of U 300 beams and filled with reinforced steel and concrete. The Frame made of anti-corrosion protected steel parts |  |  |  |
| 31.2.2 | easily accessible for servicing the system of weight sensors built into it. |  |  |  |
|  | The weighbridge shall be raised from an elevation of + - 0.00 (asphalt elevation). |  |  |  |
|  | The weighbridge shall have the following measurement and accuracy range: |  |  |  |
|  | weighting capacity from 0 kg to 50 000 kg |  |  |  |
|  | max 0.5% deviation |  |  |  |
|  | smallest notch (smallest weight increments at 20 kg max), |  |  |  |
|  | class (III) |  |  |  |
| 31.2.3 | Steel profiles for the frame of the weighbridge structure shall be dimensioned at least with profiles U-300 or more |  |  |  |
| 31.2.4 | operation in a temperature range of -10°C to +40°C and resistant to rain, snow and other natural influences |  |  |  |
| 31.2.5 | the indicator shall be resistant to temperatures from -10°C to +40°C. |  |  |  |
| 31.3 | Construction part: |  |  |  |
| 31.3.1 | Foundation: |  |  |  |
| 31.3.1.1 | Reinforced concrete pit with a service entrance with minimum C30 concrete quality |  |  |  |
| 31.3.1.2 | with strengthened reinforced concrete places where the scale rests on the ground |  |  |  |
| 31.3.1.3 | with a slight slope for atmospheric precipitation and atmospheric drainage |  |  |  |
| 31.3.1.4 | Concrete footings shall be made with minimum C30 concrete quality under the measuring cells |  |  |  |
| 31.3.2 | System of weight sensors: |  |  |  |
| 31.3.2.1 | The system shall consist of at least 8 weight sensors. |  |  |  |
| 31.3.2.2 | the sensors must be of class C3 with a load capacity of 30 t |  |  |  |
| 31.3.2.3 | sensors must have an overload capacity of 150%FS |  |  |  |
| 31.4 | The level of protection for sensors: |  |  |  |
| 31.4.1 | no less than MKS EN 60529:2006 IP68 or equivalent |  |  |  |
| 31.4.2 | cables insulated from electromagnetic influences and mechanical impacts |  |  |  |
| 31.4.3 | indicator: |  |  |  |
| 31.4. | with minimum precision of 3000 intervals |  |  |  |
| 31.4.5 | has a vibration filter |  |  |  |
| 31.4.6 | With a LED or LCD screen, showing measurement and information |  |  |  |
| 31.4.7 | Connection port RS232 compliant with the frequency at which the Customs Administration’s system operates (2400 Megahertz); or equivalent, fully interoperable with the Customs Administration’s system |  |  |  |
| 31.4.8 | power supply of 220-230 VAC at 50Hz |  |  |  |
| 31.4.9 | operating conditions within the range of at least from -10°C to +40°C, resistant to rain, snow and other natural influences, |  |  |  |
| 31.5 | Weighbridge Dismantiling, Relocation. and Re-Installation and Adjustment: |  |  |  |
| 31.5.1 | Dismantling of the existing weighbridge |  |  |  |
| 31.5.2 | Relocation of the weighbridge to a location within a distance of up to 15 km |  |  |  |
| 31.5.3 | Installation of the weighbridge |  |  |  |
| 31.5.4 | Adjustment of the existing foundations for installation of the new weighbridge |  |  |  |
| 31.6 | The following document must be issued for the scales: |  |  |  |
| 31.6.1 | verification document issued by the Bureau of metrology |  |  |  |
| 31.6.2 | calibration certificate according to MKC EN ISO 17025 |  |  |  |
| 32 | Training |  |  |  |
| 32.1 | The Contractor shall provide training for maintenance and administration of the offered equipment for minimum 6 employees in the Customs Administration, with the minimum duration of 30 working days. Training must be in English or Macedonian language. Upon completion of the training those minimum 6 persons must gain certification for completed training. |  |  |  |
| 32.2 | The contractor shall provide one engineer per delivery location for field training for the period of 30 days. |  |  |  |
| 32.3 | If the offered equipment is from a different manufacturer than the existing VMS, the contractor must also provide: Training certificates for 6 persons employed by the bidder for the following modules already installed in the VMS: - Video Management Module - Facial Recognition module - Business intelligence report module - ANPR module - Smart Wall Module |  |  |  |
| 32.4 | The Contractor shall provide training materials in English or Macedonian language for installation and configuration. |  |  |  |
| 33 | Warranty |  |  |  |
| 33.1 | 24-month commercial warranty (additional 12 months on top of the 12 months’ warranty stipulated in the General conditions art. 32.7). |  |  |  |